# VOLUME 3

# TECHNICAL SPECIFICATIONS

Technical specification is an integral part of tender documents together with general and specific terms of the contract, and other contract and project documents, and as such is considered binding during the project implementation.

GENERAL REQUIREMENTS

1. Standards

As a minimum the standards must always be met, as well as regulations and legislation of the Republic of Serbia. Other internationally recognized standards can be used only if they are equal to or more stringent compared to the corresponding Serbian standards or European technical approvals.

2. Control of the works

2.1 The Contracting Authority hires Supervision which shall, in accordance with the Law on Planning and Construction and other regulations related to this area, supervise the construction works and installation of equipment, and undertake all necessary measures that are defined by the law regulations of the Republic of Serbia, and the general and special conditions of this contract. The Contractor is required to provide at any time professional Supervision at the processes of construction, production and installation, as well as all the necessary documents and proof of origin of goods, as well as to comply with the order of professional Supervision regarding the correction of deficiencies. If Supervision finds serious deficiencies, errors or omissions the Contractor will be notified of the gross violation of the Contract and suspension of work.

2.2 The Contractor is required to provide adequate machinery, additional equipment and tools for the efficient implementation of work in accordance with the planned pace of construction and the anticipated deadline in tender documents. If Supervision determines insufficient capacity and efficiency of machinery and equipment which threaten the process of building, the Contractor might be ordered to engage additional machinery.

2.3 Protection of existing utility facilities

The Contractor takes over full responsibility for the protection of all roads and utility facilities located in the area of the site regardless of whether they are public or private property and shown in the drawings. The Contractor is required to pay special attention to these objects in order to avoid damages, otherwise takes the responsibility and the cost for repairing the damage.

2.4 Safety, handling and storage of materials and equipment

While implementing the activities at the site and around it abide the Serbian Law on Safety and Health at Work, which regulates the enhancement and implementation of health and safety at work of persons participating in work processes as well as persons in the working environment, and general and specific terms and conditions of this Contract related to the issue. 
All building materials and equipment delivered to the site for installation and assembly shall be stored so as to prevent injury of any kind of persons working at the construction site, as well as persons who happen to be on site, and also to prevent damage to them. If the Supervision establishes any damage of materials or equipment to the extent that they are not suitable for installation, the Contractor will be warned to remove them from the site and replace them at the expense of the Contractor.

The material originated from demolition or dismantling must be removed from the site as soon as possible to avoid potential injury to persons or damage to materials that should be installed.

After completion of the works the Contractor is obliged to remove all the waste generated during the construction and the built-in surplus materials, work equipment, and to bring back the site and the area around the site used to its original state.

2.5 The origin of materials, equipment and plants

All the installed material must come from EU member states, or countries covered by the Interreg-IPA CBC Bulgaria - Serbia Programme 2014-2020. However, the total value of goods (foreseen on-site) that does not exceed € 100,000 can be purchased in any country. The material coming from sources that are not approved by the Investor and technical supervision of investors can’t be used for work. Adoption of origin does not mean that all the material is approved in the source. The Contractor must ensure that only material that complies with the requirements set out in the various clauses of the specification will be used for work.

2.6 Protection of the environment

At the construction site machinery and equipment of the new generation with the right devices for exhaust gases, and whose engines meet environmental requirements that are currently on-going in the world, will be used. Take all measures and activities in accordance with national and European legislation regarding the protection of the environment. All waste and packaging that may affect the environment should be transported from the site on the same day. 
After completion of the work the Contractor will remove any surplus material and waste generated during the construction.

2.7 The responsibility of the Contractor

Any consent and authorization given by the Supervision do not relieve the Contractor of its obligations and responsibilities under the Agreement and the tender documents in any way.

3. Administrative specification

3.1 Programme implementation

Programme implementation of all construction works, works on installation of facilities and equipment must be in accordance with the implementation of the project and the activities planned in it. The Programme will include all the activities in the predicted works, and shall be submitted no later than the 5th of the month. The Programme submitted to the Supervision will contain an overview of all the activities carried out in relation to the pre-planned activities in the previous month and plan activities for the next month.

3.2 Working meetings

Investor, Supervision and Contractor will adopt a plan of having working meetings for successful monitoring and implementation of the project. These meetings will be held at least once a month, and if necessary emergency meetings can be scheduled in case of extraordinary circumstances.

3.3 Quality Assurance System

The Contractor will run a quality assurance system as evidence of compliance with the requirements of the Contract. The system must be in accordance with the details stated in the Contract, and Supervision will have the right to inspect and control all aspects of the system.

4. Implementation of works

4.1 Materials

The Contractor is obliged to procure and use only materials that are new products to perform all types of works under the Contract and in accordance with the Contract. Installing used materials and equipment will not be accepted.

Supply of all materials and equipment necessary for construction - installation and works on installation of equipment in accordance with the approved investment project is the responsibility of the Contractor. Any change in the approved project must be coordinated and approved by the Contracting Authority, upon written request and explanation. 
All materials to be used in the construction must be accompanied by the corresponding certificates of origin and quality-certificates, instructions for use and the statement confirming the compliance of each of the implanted materials and products with the essential requirements for construction works in accordance with the technical regulations and standards and other laws and bylaws related to this type of work and materials.

4.2 Tests and examinations

All tests, material and equipment testing in accordance with the Contract and the Serbian Law on Planning and Construction and other legislation and regulations, are required. 
The tests, measurement and testing of materials and completed construction works should be carried out by accredited laboratories and certified protocols. The Contracting Authority may require additional tests.

In addition to the tests listed in this specification and the Contract, the Contracting Authority may require additional tests of materials and equipment to identify any hidden defects and costs of these tests will be entirely at the expense of the Contractor if such mistakes are confirmed.

4.3 Inspection and works measurement

During the work on the construction, procurement of equipment and installation, the Contractor is required to provide all supporting documents and, in accordance with the legislation, to keep a construction diary daily signed by Responsible Contractor and Supervision and to keep the Inspection Book and to show the quantity of performed works through the Building Log Book and the Situation for the works.

During the implementation of works the Contracting Authority may, at any time, check the quality and quantity of the works done. If that cannot be done with the help of the Contractor, the Contracting Authority is entitled to engage a third party - an expert from the side, and the Contractor will bear the costs.

5. Preparation works

5.1 Marking of the building site

Before the building starts, the investor provides: marking of the building lot, regulation, leveling and construction lines, in accordance with regulations governing the construction works; marking the site with an adequate panel, consisting of: data on the facility to be constructed, the investor, the designer, the number of building permits, the contractor, start of construction and deadline for completion of construction.

The Contracting Authority and the Contractor make a record of the adopted geodetic marking. 
The Contractor is responsible for maintenance of geodetic data and table in good condition.

5.2 Location development site

The Contractor is required to submit for approval the plan of the site arrangement and enclose the construction site with mobile fence for site which is at least 2m tall.

5.3 Temporary buildings

The contractor sets on his own expense all necessary temporary facilities for their needs and the needs of the works. After completion of the works, the Contractor is obliged to remove them.

5.4 Supply

The use and supply of electricity for the needs of the site and construction is the responsibility of the Contractor. After completion of the work all the cables and all the temporary installations must be removed.

Drinking water supply and the construction site supply is the responsibility of the Contractor. Predict toilets, depending on the need, in the form of mobile sanitary systems. Supply and installation are obligations of the Contractor.

6. Dates of commencement and completion of works and takeover of performed works

The Contractor is obliged to start implementation of works from the Contract immediately upon introduction into the business by Supervision and to carry them out according the technical specifications and bill of works and hand them over to the Contracting Authority on time. 
The date of its introduction into the professional supervision is registered in the building log book, and it will be considered that the introduction of the work was carried out with cumulative fulfilment of these conditions:

- The Contracting Authority handed the Contractor the building permit and registration papers in accordance with the Serbian Law on Planning and Construction.

- The Contracting Authority provided unrestricted access to the site.

- The Contracting Authority determined professional supervision during the construction of the facility in accordance with Article 153 of the Serbian Law on Planning and Construction.

- The Contractor handed the decision on responsible Contractor.

The deadline for completion of works is the day of the minutes on takeover the Contract works and technical acceptance, and is recorded in the building log book by professional supervision. 
The Contractor notifies the Contracting Authority and the Supervision on the completion of work in writing, and the date of completion of the works is recorded in the building log book. The takeover of work is done by the Commission no later than 15 days after completion. The Commission for the transfer of works is composed of one representative of the Contracting Authority, the Contractor and the Supervision, and the same keep record on the takeover. Any errors or defects determined by the Contracting Authority during takeover of the works, the Contractor shall be obliged to remove without delay, and if the Contractor do not begin their removal within three days or if it fails to remove them within an agreed deadline, the Contracting Authority will entrust the work to some other contractor at the expense of the Contractor, i.e. collection of collaterals for good performance. Possible transfer of eliminating defects to other contractor, the Contracting Authority will make at market prices and with due diligence.

The Contractor is obliged to enclose a study surveying works for the object during takeover and as part of a special study surveying works for storm sewers and water mains. Study surveying works should be done by a company licensed for this type of work, signed and certified in a legal way.

The contractor is obliged to hand over the project implementation in two copies during the takeover, if they made changes to the project documentation.

The final record of the technical approval and acceptance of the work performed is kept after correction of identified defects.

SPECIAL REQUIREMENTS

The preliminary design planned arrangement of vacated land with assembling of urban equipment and construction of path next to Toponica river on cadastre parcel 4119 KO Vele Polje and also access paths and paths to tower Zeleznik on cadastre parcel 4082/1 KO Vele Polje. Beneficiaries on these two parcels are PWC “Srbija vode” and PC “Srbijasume”, which gave written consent for preliminary design and realization of project in question.

City municipality Crveni Krst received a Notification from Directorate for Planning and Construction, No 351-482/15-06, dated 29th July 2015, which for this kind of construction works and works on assembling this kind of objects is not required to acquire Act for construction works. In November, Municipality Crveni Krst chose one geodetic company, based on two public procurements, for geodetic survey of terrain and a design company for making of preliminary design for construction of pedestrian paths, all with a goal of getting the most accurate data needed for production of Bill of Quantities.

Conceptual designs within preliminary designs include:

1.	Construction of access pedestrian path from village Miljkovac to Vidriste on cadastre parcel 4082/1 KO Vele Polje, which designed length is 883,84m1. Projected width of pedestrian path is 3m, with shoulder wide 0.5m on the right and drain flume on the left. With this Project, it is planned to construct three concrete culverts Ø800, because of hilly terrain and for draining water from path with drain flume. Maximal grade of finished level is 13,78%. Tampon from stone aggregate and pebble is planned for lower carrying layer, thick 10+15cm and concrete surfacing thick 7cm.

2.	Construction of pedestrian path out of rural road to the tower “Zeleznik” on cadastre parcel 4082/1 KO Vele Polje, designed length 128,30m, that is located in extremely mountainous  part of the vacationland where the maximal longitudinal tilt is 38,31%. Environment in which the construction of path is being done, III and IV category of land is recognized. Width of the path is 1,2m and because of the configuration of the terrain a metal fence is planned from both sides in upper part of the path and only on one side on the lower part of the path. It is planned that the path is made from stone road surface and concrete MB 30 as finishing layer in the form of a staircase – as seen in longitudinal profile. Draining of the road is entirely gravitationally. During concreting path ,reionforced with steel fabric Q 196

3.	Construction of pedestrian path by the Toponica river on cadastre parcel 4119 KO Vele Polje – designed length is 478,96. Route of designed pedestrian path stretches on existing route and follows Toponica riverbed. Trail is located, largely, on lowland part of the terrain, and maximal longitudinal tilt is 7,69%. Geometry of route is done based on record of existing conditions. Draining of the path is done with longitudinal and transverse tilts with which the water is being drain away to Toponica river. Width of the path is 1.2m and construction out of stamped concrete, thick 10cm, is planned and crushed stone aggregate and pebble thick 10+20cm is planned as lower baring layer.

Arranging of vacationland is planned as presented in preliminary design, along pedestrian street by Toponica river, along existing rural path that connects this path with the path that leads to tower Zeleznik. Minimal construction works are planned so that the natural environment can be preserved, and because of that, in the place where the urban equipment is going to be assembled, it is not planned to build concrete plateaus but only building single foundation for which only bearing parts of the structure are being anchored.  Also, it is not planned to cut and remove of wood but, eventually, cutting of undergrowth and overgrowth that would be in the way while construction works. All technical characteristic for assembling and production of urban equipment are given in drawings and detailed bill of quantities.

The Project planned to arrange following urban equipment:

- Hexagonal wooden marquee made out of firm wood, dimensions 2.40x2.60, with fence on which the benches are leaning on and table in the middle. Marquee roof is made out of processed boards and all construction is being anchored trough poles by concreting into the base. Roof also can be made from impregnated wooden shingle, with previous agreement from the Supervisor and Contracting Authority. It is planned to place 22 marquees that are coated with coating for wood impregnation and painted in two layers with wood coating in color chosen by the investor. Height of the marquee is 3.50m.

- Round picnic table for 8 persons, monolithic produced – attached to benches with body of the table, made out of fir wood, thick 5cm, according to everything on the drawing. The outer diameter is 2m, and the diameter of the table is 1.1m. Height of the table is 72cm and binding of 4 foundations is planned to concrete footings trough anchorages. For tables with benches, the same protection of the wood like with marquees is planned. The project foresees placing 8 tables on those parts of vacationland where there is already natural shade from the surrounding woods or trees by the river.

- Placement of wooden benches is planned in 18 places by the path for resting of the pedestrians. Benches will be made, in full, out of wooden boards, thick 5cm, in all according to the drawing and in two places anchored to concrete foundations. Protection of the wood is the same as in previously mentioned urban equipment.

- Information board on which will, among other things, will be shown a map of the vacationland and access paths will be set in two places – at the entrance of the vacationland and next to the path leading to tower Zeleznik. Construction of the board and the roof will be made out of gum pole, and the board itself and the roof out of sawn wood. Wood will be protected with coating for wood impregnation and protective color layers and connected with anchorages to two foundations.

- Sales wooden stalls, made out of sawn wood and boards thick 5cm, in all according to sketch, for selling healthy food, souvenirs, promotion of old crafts etc. It is planned to set 2 stalls by the path that goes by the Toponica river. They will be hitched with anchorages to the foundations; wood protection is the same like in abovementioned cases.

- Waste baskets are made out of metal construction, and the basket part is covered with wooden laths, dimensions 0.3х0.5, processed and protected according to mentioned procedure. Insole for the basket is foreseen to be metal, but that can be removed for cleaning of the basket. Attaching for foundation is trough hard faced anchorage board. Metal construction and wooden coverage are painted and protected according to investor’s order.  It is planned to place 25 waste baskets.

- Placing of road signs in vacationland will be in cooperation with company that controls the road. It is needed to acquire conditions for setting, consents, make the signs in authorized company and attach them in place where the controller of the road determinates. Placing of 3 road signs is planned.

- For vacationland to be completely equipped and adapted to need, it is planned placement of poles with solar lighting along access path and along paths in vacationland itself. Because of the specificity of the purchase and installation of this type of lighting , it is planned to purchase the poles from companies that are registered for staging and montage of itself and it is planned to purchase them with complete equipment: pole height 4m with battery and panels carrier, solar panel 140W, controller 20A, battery 120 Ah vrla gel, LED reflector. Based on the above mentioned characteristics and needs in field it is planned to place 30 poles with solar lighting.

- With goal of preserving of built paths and urban equipment, because of the investments of higher value, it is planned to install video surveillance with 9 cameras and complete following equipment which detailed specification is given in Bill of quantities. Cameras will be placed along the path and by the tower “Zeleznik”. Also, with this position is planned setting up routers and complete equipment for wireless internet.

- With this project it is planned setting up 2 billboards on which will be visibly highlighted data about Project and investors on the project, with which is provided visualization of the project, and one will be placed in center of village Miljkovac, and the other one in vacationland. For building and montage of billboards wooden construction is planned - frame is from wooden beams 14x14, tied to concrete footings with in advance concreted anchorages, placing the data

All works must be performed in accordance with technical regulations and standards and other applicable legislation. Works on construction of footpaths must comply with parts of the text documents of the Project: Technical Description, General Technical Requirements for Implementation of Works, Safety at Work, Environmental Protection.

By Decision No.020-152 / 2 obtained from the Institute for Nature Conservation of Serbia the following conditions of nature protection are given:

- Planned furniture should be made of natural materials (wood) and fit into the surrounding environment visually.

- Planned protection and complete impregnation of the wooden structure of furniture should be done off-site with corresponding certified compositions.

- Area arrangement and the subject premises set up to perform with the least possible influence on the surrounding area and its environmental values.

- It is not allowed to use striking colours for finishing the surfaces of the subject premises.

- It is not allowed to cut high vegetation for the purpose of furniture setting.

- Footpaths arrangement should be carried out with the least possible influence on the surrounding area and its natural and environmental values.

- The earth works should be carried out so as to avoid over-excavation of the field, except as it is necessary.

- The path passing through the bushy vegetation should be cleaned of existing vegetation and cut vegetation should be carried away from the path.

- It is allowed to use geological building material from the cut out route (kerf or slit).

- Path width should be done according to the Project.

- The groundwork of paths should be well beaten earth.

- On the path of well beaten earth from Miljkovac to Vidrište a layer of gravel and stone aggregate will be put and the road way will be of asphalt.

- To build the path to the tower Železnik of stone overspread and concrete steps.

- To build the path along the river Toponica of stone aggregate and gravel, as well as finishing of printed concrete.

- The paths should be built with slope on both sides due to drain rainwater. If this is not possible it is necessary to install the drains for surface waters that are placed along the edge of the slit by the path.

- Surface layer of soil, humus, which will be removed during the works, is necessary to be saved and used later for the terrain repair.

- The removal of vegetation by burning is forbidden.

- To provide protection of single trees and groups of trees near the implementation of the activities, which may be adversely affected during the operation of building machines, means of transport or storage equipment.

- To take all measures against soil erosion when planning and stabilize the soil in order to prevent its collapse or slip. In case that during the implementation of the planned works occur instability of ground the Contractor is obliged by order of Investor to take urgent and appropriate measures for the repair of the site.

- It is not allowed to use water from the Toponica river for the purposes of implementation of works.

- Disposal and discharge of solid and liquid waste in the Toponica river is strictly forbidden.

- For area lighting to apply appropriate technical solutions in accordance with the function of location.

- It is not allowed to use powerful light sources directed towards the sky (rotating lasers, spotlights, etc.).

- To define and provide a temporary location for the storage of necessary materials and equipment, as well as for the storage of waste materials and their evacuation by the relevant municipal departments.

- Use native plant species for biological re-cultivation.

As the "Zeleznik" Tower, where the path No. 2 extends, was declared a cultural monument in 1986, the Institute for Protection of Cultural Monuments of Niš made the Decision No.859 / 2, on establishing the conditions for undertaking technical protection measures for infrastructural development of locations in the immediate environment of "Zeleznik" Tower in order to protect the site from damage, as well as to provide conditions for the protection of newly found object or objects.

Tenderers are advised to, due to the specific conditions of building, make sure to visit the location where the work is implemented.

Based on the Regulation on the minimum warranty period for certain types of buildings and works, the minimum warranty period for the works is 2 years from the date of handover of the works.